REUSE MANUAL

USNG

10xxxxxx.1

Implementation

# Table of Contents

Table of Contents	i

SECTION 1.  Introduction	1

1.1	Purpose of the reuse Manual	1

1.2	PURPOSE OF THE REUSABLE SOFTWARE COMPONENT	1

1.3	GENERAL INFORMATION	2

1.3.1	POINT OF CONTACT	2

1.3.2	CERTIFICATION LEVEL	2

1.3.3	LEGAL RESTRICTIONS	2

Section 2.  INSTALLATION	3

2.1  PARTIAL REUSE	3

2.2  MODIFICATIONS	3

Section 3.  ENVIRONMENT	4

3.1	HARDWARE	4

3.1.1	DEVELOPMENT	4

3.1.2	TARGET	4

3.2	SOFTWARE	4

3.2.1	OPERATING SYSTEM	4

3.2.2	COMPILERS	4

3.3	ASSUMPTIONS AND PERFORMANCE LIMITATIONS	5

Section 4.  Global RSC Environment	6

4.1	TYPES	6

4.2	CONSTANTS	6

4.3	VARIABLES	6

4.4	INCLUDE FILES	6

4.5	dependencies	6

SECTION  5.  FUNCTIONS	8

5.1	SET_USNG_PARAMETERS	8

5.1.1	DESCRIPTION	8

5.1.2	INTERFACES AND EXAMPLES	8

5.1.3	DECLARATIONS	8

5.1.4	DEPENDENCIES	9

5.1.5	ERROR HANDLING	9

5.2	GET_USNG_PARAMETERS	9

5.2.1	DESCRIPTION	9

5.2.2	INTERFACES AND EXAMPLES	9

5.2.3	DECLARATIONS	10

5.2.4	DEPENDENCIES	10

5.2.5	ERROR HANDLING	10

5.3	Convert_Geodetic_To_USNG	10

5.3.1	DESCRIPTION	10

5.3.2	INTERFACES AND EXAMPLES	10

5.3.3	DECLARATIONS	11

5.3.4	DEPENDENCIES	11

5.3.5	ERROR HANDLING	12

5.4	Convert_USNG_To_Geodetic	12

5.4.1	DESCRIPTION	12

5.4.2	INTERFACES AND EXAMPLES	12

5.4.3	DECLARATIONS	13

5.4.4	DEPENDENCIES	13

5.4.5	ERROR HANDLING	14

5.5	Convert_UTM_To_USNG	14

5.5.1	DESCRIPTION	14

5.5.2	INTERFACES AND EXAMPLES	14

5.5.3	DECLARATIONS	15

5.5.4	DEPENDENCIES	15

5.5.5	ERROR HANDLING	16

5.6	Convert_USNG_To_UTM	16

5.6.1	DESCRIPTION	16

5.6.2	INTERFACES AND EXAMPLES	16

5.6.3	DECLARATIONS	17

5.6.4	DEPENDENCIES	17

5.6.5	ERROR HANDLING	17

5.7	Convert_UPS_To_USNG	18

5.7.1	DESCRIPTION	18

5.7.2	INTERFACES AND EXAMPLES	18

5.7.3	DECLARATIONS	19

5.7.4	DEPENDENCIES	19

5.7.5	ERROR HANDLING	19

5.8	Convert_USNG_To_UPS	19

5.8.1	DESCRIPTION	19

5.8.2	INTERFACES AND EXAMPLES	19

5.8.3	DECLARATIONS	20

5.8.4	DEPENDENCIES	21

5.6.5	ERROR HANDLING	21

APPENDIX A  STRUCTURE/DEPENDENCY DIAGRAMS	22

APPENDIX B  DEFINITIONS/GLOSSARY	23

APPENDIX C  REFERENCES	24

# SECTION 1.  Introduction

## 1.1	Purpose of the reuse Manual

This document describes the characteristics of the USNG reusable software component and provides instructions on its installation and operation.  The manual is a self-contained reference for the software engineer intending to incorporate the component in another software system.  This manual was written with the assumption that the user has a basic working knowledge of C and is familiar with fundamental C concepts and terminology.

## 1.2	PURPOSE OF THE REUSABLE SOFTWARE COMPONENT

The purpose of USNG is to provide a reusable software component which supports the following coordinate conversions :

- Geodetic coordinates (latitude and longitude in radians) to United States National Grid (USNG) coordinate string, and

- United States National Grid (USNG) coordinate string to Geodetic coordinates (latitude and longitude in radians).

- Universal Transverse Mercator (UTM) coordinates (zone, hemisphere, easting and northing in meters) to USNG coordinate string.

- USNG coordinate string to Universal Transverse Mercator (UTM) coordinates (zone, hemisphere, easting and northing in meters).

- Universal Polar Stereographic (UPS) coordinates (hemisphere, easting and northing in meters) to USNG coordinate string.

- USNG coordinate string to Universal Polar Stereographic (UPS) coordinates (hemisphere, easting and northing in meters).

- A particular ellipsoid is specified in terms of the following parameters:

- Semi-Major Axis (a):	Radius (in meters) at the equator,

- Flattening (f):	Ratio of the difference between the semi-major axis and polar    radius of the Earth to its semi-major axis.

- Ellipsoid Code		Standard 2-letter ellipsoid code.

## 1.3	GENERAL INFORMATION

### 1.3.1	POINT OF CONTACT

- U.S. Army Topographic Engineering Center (USATEC)

- Geospatial Information Division (GID)

- ATTN:  CETEC-GD-A (Dan Specht)

- 7701 Telegraph Road

- Alexandria, VA  22315-3864

- Dan Specht		(703) 428 - 6761	Project Manager

### 1.3.2	CERTIFICATION LEVEL

- This RSC has been certified at level 4.  A level 4 component satisfies the criteria for reliability, testing, and documentation for the Army Reuse Center (ARC).  The component comes with test materials and a Reuse Manual that aids in integrating the component into a software system.

### 1.3.3	LEGAL RESTRICTIONS

- This Reusable Software Component (RSC) contains data with Unlimited Government Rights.

# Section 2.  INSTALLATION

- The following is a list of the files which make up the USNG component:

- Source Code Files:

- usng.c

- Header Files :

- usng.h

- Data Files :

- none

- The compilation instructions for the USNG component are as follows:

- DOS Makefile (Uses Microsoft C):

- cl /nologo /W3 /FR /G2 /DNDEBUG /Gs /Ox /AM /D_DOS /c usng.c

- UNIX Makefile (Uses gcc compiler):

- cc -g -O -ansi -Wall -c usng.c

- The compilation order of the USNG component relative to other components is unconstrained.

## 2.1  PARTIAL REUSE

- The USNG component does not allow for partial reuse.

## 2.2  MODIFICATIONS

- The USNG component does not permit modifications.

# Section 3.  ENVIRONMENT

- This section provides details on the environment under which USNG was developed, tested, and executed.

## 3.1	HARDWARE

### 3.1.1	DEVELOPMENT

- The following is a list of hardware configurations under which USNG was developed and tested.

- SUN SparcStation 20

- IBM compatible Pentium PC

### 3.1.2	TARGET

- The following is a list of hardware configurations under which USNG was executed.

- SUN SparcStation 20

- IBM compatible Pentium PC

## 3.2	SOFTWARE

### 3.2.1	OPERATING SYSTEM

- The following is a list of operating systems under which USNG was executed and tested.

- Solaris 2.7

- Windows XP

### 3.2.2	COMPILERS

- The following is a list of compilers on which USNG was compiled successfully.

- GCC version 2.95.2

- Microsoft Visual C++ version 6

## 3.3	ASSUMPTIONS AND PERFORMANCE LIMITATIONS

There are no hardware or environment constraints.  There are no limitations.

This RSC is written in ANSI C.

# Section 4.  Global RSC Environment

## 4.1	TYPES

Not applicable.

## 4.2	CONSTANTS

The following is a list of significant visible constants declared globally in USNG with their descriptions.

USNG_NO_ERROR		: No errors occurred in function

USNG_LAT_ERROR		: Latitude outside of valid range

(-90 to 90 degrees)

USNG_LON_ERROR		: Longitude outside of valid range

(-180 to 360 degrees)

USNG_STRING_ERROR		: A USNG string error: string too long, string too short, or badly formed

USNG_PRECISION_ERROR		: The precision must be between 0 and 5 inclusive.

USNG_A_ERROR		: Semi-major axis less than or equal to zero

USNG_INV_F_ERROR		: Inverse flattening outside of valid range

(250 to 350)

USNG_EASTING_ERROR		: Easting outside of valid range

USNG_NORTHING_ERROR		: Northing outside of valid range

USNG_ZONE_ERROR		: Zone outside of valid range (1 to 60)

USNG_HEMISPHERE_ERROR		: Invalid hemisphere (‘N’ or ‘S’)

## 4.3	VARIABLES

Not applicable.

## 4.4	INCLUDE FILES

<ctype.h>	: Standard C character handing library

<math.h>	: Standard C math library

<stdio.h>	: Standard C input/output library

<string.h>	: Standard C string handling library

ups.h	: Used to convert between geodetic and UPS

utm.h	: Used to convert between geodetic and UTM

usng.h	: Error codes and prototype error checking

## 4.5	dependencies

The following is a list of the software external to the RSC and its descriptions.

- UTM, since USNG coordinates are based on Universal Transverse Mercator (UTM) coordinates in all but the polar regions.

- UPS, since USNG coordinates are based on Universal Polar Stereographic (UPS) coordinates in the polar regions.

# SECTION  5.  FUNCTIONS

## 5.1	SET_USNG_PARAMETERS

### 5.1.1	DESCRIPTION

This function sets the ellipsoid parameters to the specified values.

### 5.1.2	INTERFACES AND EXAMPLES

The following is a list of the formal arguments required to use this function.

- long Set_USNG_Parameters (double a,

- double f,

- char *Ellipsoid_Code);

- a	Semi-major axis of ellipsoid in meters (input),

- f	Flattening of ellipsoid (input),

- Ellipsoid_Code	2 letter code for ellipsoid (input).

Example:

- status = Set_USNG_Parameters (a, f, Ellipsoid_Code)

Inputs:

- a	6378137.0

- f	1 / 298.25722210100

- Ellipsoid_Code 	“RF”

Outputs:

- None.

### 5.1.3	DECLARATIONS

#### 5.1.3.1	TYPES

Not applicable.

#### 5.1.3.2	CONSTANTS

Not applicable.

#### 5.1.3.3	VARIABLES

Not applicable.

### 5.1.4	DEPENDENCIES

- None.

### 5.1.5	ERROR HANDLING

This function returns the following status codes:

USNG_NO_ERROR		: No errors occurred in function

USNG_A_ERROR		: Semi-major axis less than or equal to zero

USNG_INV_F_ERROR		: Inverse flattening outside of valid range

(250 to 350)

## 5.2	GET_USNG_PARAMETERS

### 5.2.1	DESCRIPTION

This function returns the current values of the ellipsoid parameters.

### 5.2.2	INTERFACES AND EXAMPLES

The following is a list of the formal arguments required to use this function.

- void Get_USNG_Parameters (double *a,

- double *f,

- char *Ellipsoid_Code);

- a	Semi-major axis of ellipsoid in meters (output),

- f	Flattening of ellipsoid (output),

- Ellipsoid_Code	2 letter code for ellipsoid (output).

### 5.2.3	DECLARATIONS

#### 5.2.3.1	TYPES

Not applicable.

#### 5.2.3.2	CONSTANTS

Not applicable.

#### 5.2.3.3	VARIABLES

Not applicable.

### 5.2.4	DEPENDENCIES

None.

### 5.2.5	ERROR HANDLING

No errors are reported by this function.

## 5.3	Convert_Geodetic_To_USNG

### 5.3.1	DESCRIPTION

This function converts Geodetic coordinates (latitude and longitude in radians) to a USNG coordinate string with the specified level of precision.

### 5.3.2	INTERFACES AND EXAMPLES

The following is a list of the formal arguments required to use this function.

- long Convert_Geodetic_To_USNG (double Latitude,

- double Longitude,

- long Precision,

- char * USNG_String)

- Latitude	Latitude in radians (input),

- Longitude	Longitude in radians (input),

- Precision	Level of precision (number of digits) for USNG string (input),

- USNG_String	USNG coordinate string (output).

Example:

- status = Convert_Geodetic_To_USNG (Latitude, Longitude, Precision, USNG_String)

Inputs:

- Latitude:	35.0

- Longitude:	-75.0

- Precision	5

Outputs:

- USNG_String:	18SWD0000073043

### 5.3.3	DECLARATIONS

#### 5.3.3.1	TYPES

Not applicable.

#### 5.3.3.2	CONSTANTS

Not applicable.

#### 5.3.3.3	VARIABLES

Not applicable.

### 5.3.4	DEPENDENCIES

- Set_UTM_Parameters, in the UTM component – used to set the parameters of the Universal Transverse Mercator (UTM) projection, and

- Convert_Geodetic_To_UTM, in the UTM component – used to convert geodetic (latitude and longitude) coordinates to Universal Transverse Mercator (UTM) projection (hemisphere, zone, easting and northing) coordinates.

- Set_UPS_Parameters, in the UPS component – used to set the parameters of the Universal Polar Stereographic (UPS) projection, and

- Convert_Geodetic_To_UPS, in the UPS component – used to convert geodetic (latitude and longitude) coordinates to Universal Polar Stereographic (UPS) projection (hemisphere, easting and northing) coordinates.

### 5.3.5	ERROR HANDLING

This function returns the following status codes:

USNG_NO_ERROR		: No errors occurred in function

USNG_LAT_ERROR		: Latitude outside of valid range

(-90 to 90 degrees)

USNG_LON_ERROR		: Longitude outside of valid range

(-180 to 360 degrees)

USNG_PRECISION_ERROR		: The precision must be between 0 and 5 inclusive.

## 5.4	Convert_USNG_To_Geodetic

### 5.4.1	DESCRIPTION

This function converts a USNG coordinate string to Geodetic coordinates (latitude and longitude in radians).

### 5.4.2	INTERFACES AND EXAMPLES

The following is a list of the formal arguments required to use this function.

- long Convert_USNG_To_Geodetic (char* USNG_String,

- double* Latitude,

- double* Longitude);

- USNG_String	USNG coordinate string (input),

- Latitude	Latitude in radians (output),

- Longitude	Longitude in radians (output).

Example:

- status = Convert_USNG_To_Geodetic (USNG_String, &Latitude, &Longitude)

Inputs:

- USNG_String:	18SWD0000073043

Outputs:

- Latitude:	35.0

- Longitude:	-75.0

### 5.4.3	DECLARATIONS

#### 5.4.3.1	TYPES

Not applicable.

#### 5.4.3.2	CONSTANTS

Not applicable.

#### 5.4.3.3	VARIABLES

Not applicable.

### 5.4.4	DEPENDENCIES

- Set_UTM_Parameters, in the UTM component – used to set the parameters of the Universal Transverse Mercator (UTM) projection, and

- Convert_UTM_To_Geodetic, in the UTM component – used to convert Universal Transverse Mercator (UTM) projection (hemisphere, zone, easting and northing) coordinates to geodetic (latitude and longitude) coordinates.

- Set_UPS_Parameters, in the UPS component – used to set the parameters of the Universal Polar Stereographic (UPS) projection, and

- Convert_UPS_To_Geodetic, in the UPS component – used to convert Universal Polar Stereographic (UPS) projection (hemisphere, easting and northing) coordinates to geodetic (latitude and longitude) coordinates.

### 5.4.5	ERROR HANDLING

This function returns the following status codes:

USNG_NO_ERROR		: No errors occurred in function

USNG_STR_ERROR		: An USNG string error: string too long, string too short, or badly formed.

## 5.5	Convert_UTM_To_USNG

### 5.5.1	DESCRIPTION

This function converts Universal Transverse Mercator (UTM) coordinates (zone, hemisphere, easting and northing in meters) to a USNG coordinate string with the specified level of precision.

### 5.5.2	INTERFACES AND EXAMPLES

The following is a list of the formal arguments required to use this function.

- long Convert_UTM_To_USNG (long Zone,

- char Hemisphere,

- double Easting,

- double Northing,

- long Precision,

- char * USNG_String)

- Zone	UTM zone (1 to 60) (input),

- Hemisphere	Northern (‘N’) or Southern (‘S’) hemisphere (input),

- Easting	Easting (X) in meters (input),

- Northing	Northing (Y) in meters (input).

- Precision	Level of precision (number of digits) for USNG string (input),

- USNG_String	USNG coordinate string (output).

Example:

- status = Convert_UTM_To_USNG (Zone, Hemisphere, Easting, Northing, Precision, USNG_String)

Inputs:

Inputs:

- Zone:	18

- Hemisphere:	‘N’

- Easting:	500000

- Northing:	3873042

- Precision	5

Outputs:

- USNG_String:	18SWD0000073042

### 5.5.3	DECLARATIONS

#### 5.5.3.1	TYPES

Not applicable.

#### 5.5.3.2	CONSTANTS

Not applicable.

#### 5.5.3.3	VARIABLES

Not applicable.

### 5.5.4	DEPENDENCIES

- Set_UTM_Parameters, in the UTM component – used to set the parameters of the Universal Transverse Mercator (UTM) projection, and

- Convert_Geodetic_To_UTM, in the UTM component – used to convert geodetic (latitude and longitude) coordinates to Universal Transverse Mercator (UTM) projection (hemisphere, zone, easting and northing) coordinates.

### 5.5.5	ERROR HANDLING

This function returns the following status codes:

USNG_NO_ERROR		: No errors occurred in function

USNG_ZONE_ERROR		: Zone outside of valid range

(1 to 60)

USNG_HEMISPHERE_ERROR		: Invalid hemisphere (‘N’ or ‘S’)

USNG_EASTING_ERROR		: Easting outside of valid range

(100,000 to 900,000m)

USNG_NORTHING_ERROR		: Northing outside of valid range

(0 to 10,000,000m)

USNG_PRECISION_ERROR		: The precision must be between 0 and 5 inclusive.

## 5.6	Convert_USNG_To_UTM

### 5.6.1	DESCRIPTION

This function converts an USNG coordinate string to Universal Transverse Mercator (UTM) projection coordinates (zone, hemisphere, easting in meters, and northing in meters).

### 5.6.2	INTERFACES AND EXAMPLES

The following is a list of the formal arguments required to use this function.

- long Convert_USNG_To_UTM (char *USNG_String,

- long   *Zone,

- char   *Hemisphere,

- double *Easting,

- double *Northing)

- USNG_String	USNG coordinate string (input),

- Zone	UTM zone (1 to 60) (output),

- Hemisphere	Northern (‘N’) or Southern (‘S’) hemisphere (output),

- Easting	Easting (X) in meters (output),

- Northing	Northing (Y) in meters (output).

Example:

- status = Convert_USNG_To_UTM (USNG_String, &Zone, &Hemisphere, &Easting, &Northing)

Inputs:

- USNG_String:	18SWD0000073042

Outputs:

- Zone:	18

- Hemisphere:	‘N’

- Easting:	500000

- Northing:	3873042

### 5.6.3	DECLARATIONS

#### 5.6.3.1	TYPES

Not applicable.

#### 5.6.3.2	CONSTANTS

Not applicable.

#### 5.6.3.3	VARIABLES

Not applicable.

### 5.6.4	DEPENDENCIES

- None.

### 5.6.5	ERROR HANDLING

This function returns the following status codes:

USNG_NO_ERROR		: No errors occurred in function

USNG_STR_ERROR		: An USNG string error: string too long, string too short, or badly formed.

## 5.7	Convert_UPS_To_USNG

### 5.7.1	DESCRIPTION

This function converts Universal Polar Stereographic (UPS) coordinates (hemisphere, easting and northing in meters) to an USNG coordinate string with the specified level of precision.

### 5.7.2	INTERFACES AND EXAMPLES

The following is a list of the formal arguments required to use this function.

- long Convert_UTM_To_USNG (char Hemisphere,

- double Easting,

- double Northing,

- long Precision,

- char *USNG_String)

- Hemisphere	Northern (‘N’) or Southern (‘S’) hemisphere (input),

- Easting	Easting (X) in meters (input),

- Northing	Northing (Y) in meters (input).

- Precision	Level of precision (number of digits) for USNG string (input),

- USNG_String	USNG coordinate string (output).

Example:

- status = Convert_UPS_To_USNG (Hemisphere, Easting, Northing, Precision, USNG_String)

Inputs:

- Hemisphere:	“S”

- Easting:	2222990

- Northing:	1797470

- Precision	5

Outputs:

- USNG_String:	BCK2299097470

### 5.7.3	DECLARATIONS

#### 5.7.3.1	TYPES

Not applicable.

#### 5.7.3.2	CONSTANTS

Not applicable.

#### 5.7.3.3	VARIABLES

Not applicable.

### 5.7.4	DEPENDENCIES

- None.

### 5.7.5	ERROR HANDLING

This function returns the following status codes:

USNG_NO_ERROR		: No errors occurred in function

USNG_HEMISPHERE_ERROR		: Invalid hemisphere (‘N’ or ‘S’)

USNG_EASTING_ERROR		: Easting outside of valid range

(0 to 4,000,000m)

USNG_NORTHING_ERROR		: Northing outside of valid range

(0 to 4,000,000m)

USNG_PRECISION_ERROR		: The precision must be between 0 and 5 inclusive.

## 5.8	Convert_USNG_To_UPS

### 5.8.1	DESCRIPTION

This function converts an USNG coordinate string to Universal Polar Stereographic (UPS) projection coordinates (hemisphere, easting in meters, and northing in meters).

### 5.8.2	INTERFACES AND EXAMPLES

The following is a list of the formal arguments required to use this function.

- long Convert_USNG_To_UPS (char *USNG_String,

- char   *Hemisphere,

- double *Easting,

- double *Northing)

- USNG_String	USNG coordinate string (input),

- Hemisphere	Northern (‘N’) or Southern (‘S’) hemisphere (output),

- Easting	Easting (X) in meters (output),

- Northing	Northing (Y) in meters (output).

Example:

- status = Convert_USNG_To_UPS (USNG_String, &Hemisphere, &Easting, &Northing);

Inputs:

- USNG_String:	BCK2299097470

Outputs:

- Hemisphere:	“S”

- Easting:	2222990

- Northing:	1797470

### 5.8.3	DECLARATIONS

#### 5.8.3.1	TYPES

Not applicable.

#### 5.8.3.2	CONSTANTS

Not applicable.

#### 5.8.3.3	VARIABLES

Not applicable.

### 5.8.4	DEPENDENCIES

- None.

### 5.6.5	ERROR HANDLING

This function returns the following status codes:

USNG_NO_ERROR		: No errors occurred in function

USNG_STR_ERROR		: An USNG string error: string to long, string to short, or badly formed.

# APPENDIX A  STRUCTURE/DEPENDENCY DIAGRAMS

This component consists of a single compilation unit and depends on both the UTM and UPS components.

USNG

Set_UPS_Parameters

Convert_UPS_To_Geodetic

Convert_Geodetic_To_UPS

UPS

UTM

Set_UTM_Parameters

Convert_UTM_To_Geodetic

Convert_Geodetic_To_UTM

# APPENDIX B  DEFINITIONS/GLOSSARY

Coordinate – Linear or angular quantities that designate the position that a point occupies in a given reference frame or system.  Also used as a general term to designate the particular kind of reference frame or system, such as Cartesian coordinates or spherical coordinates.

Ellipsoid – The surface generated by an ellipse rotating about one of its axes.

Geodetic Coordinates – The quantities of latitude and longitude that define the position of a point on the surface of the earth with respect to the reference ellipsoid.  Also, imprecisely called geographic coordinates.

Geodetic Latitude – The angle between the plane of the equator and the normal to the ellipsoid through the computation point.  Geodetic latitude is positive north of the equator and negative south of the equator.

Geodetic Longitude – The angle between the plane of a meridian and the plane of the prime meridian.  A longitude can be measured from the angle formed between the local and prime meridians at the pole of rotation of the reference ellipsoid, or by the arc along the Equator intercepted by these meridians.

USNG Coordinates – The United States National Grid (USNG) is an alphanumeric system, based on the UTM and UPS map projections, for identifying positions.  A USNG coordinate consists of a zone designation, alphabetic 100,000-meter grid square designator, and numeric coordinates within the 100,000 meter grid square.

# APPENDIX C  REFERENCES

(1)  United States National Grid (USNG), FGDC, December 2001, Reston, Virginia.